Lab 2: MIPS Assembly Programming

Introduction

In this lab you will learn to program the MIPS core by writing a MIPS assembly program to calculate the Fibonacci numbers.

MIPS Assembly Tutorial

You will use Codescape tools to compile MIPS assembly language programs and then download, and run them on the MIPS core. You will write the same Fibonacci program, but this time using MIPS assembly language.

- Example MIPS Assembly Program

Before writing your own program, first let's take a look at an example MIPS assembly language program. Browse to the Lab_2\ASM\ReadSwitches directory. Open the main.S MIPS assembly file using a text editor such as Notepad++ or Wordpad. The main.S file contains the ReadSwitches program, as also shown in Figure 1.

# $12 = address of switches, $13 = address offset of switches

# $10 = switch values

.globl main

main:

lui   $12, 0xbf80    # $12 = address of LEDs (0xbf800000)

addiu $13, $12, 4    # $13 = SW address offset

readIO:

lw    $10, 0($13)    # read switches: $10 = switch values

sw    $10, 0($12)    # write switch values to LEDs

beq   $0, $0, readIO # repeat

nop                  # branch delay slot

Figure 1. ReadSwitches MIPS assembly program

This program performs the following steps:

- Loads the memory-mapped addresses of the switches and LEDs into registers $12 and $13.

readIO:

lui   $12, 0xbf80    # $12 = address of LEDs (0xbf800000)

addiu $13, $12, 4    # $13 = address of switches

- Reads the value of the switches on the FPGA board:

lw    $10, 0($13)    # read switches: $10 = switch values

- Writes that value to the LEDs

sw    $10, 0($12)    # write switch values to LEDs

- And repeats

beq   $0, $0, readIO # repeat

nop                  # branch delay slot

The .globl main assembler directive at the top of the file is required so that the compiler can find the main function when linking the program with the boot code.

A write to memory address 0xbf800000 will output a value to the LEDs and a read to address 0xbf800008 will return the value of the switches.

- Compiling, Running, and Debugging

Now you will compile, run, and debug the ReadSwitches MIPS assembly program on the MIPS core. Each step is described in detail below.

Step 1. Download the MIPS system onto the FPGA board

Step 2. Compile the MIPS assembly program

Step 3. Load the program onto MIPS using the UART interface

- Step 1. Download the MIPS system onto the FPGA board

Refer to Lab 1 instructions if you have forgotten how to do this.

- Step 2. Compile the program

Now compile the ReadSwitches example Assembly program by opening a command window (Start menu → cmd.exe) and changing to the following directory:

MIPS_Labs\Lab_2\ASM\ReadSwitches

For example, if MIPS_Labs is located at C:\ type:

cd C:\MIPS_Labs\Lab_2\ASM\ReadSwitches

Then compile the example MIPS assembly program by typing make at the prompt in the command window:

make

This runs the Makefile to compile the code found in main.S. The executable will include not only the user code found in main.S but also the boot code, found in boot.S and the other .S files. Open and view the Makefile using a text editor such as Notepad++. The Makefile is almost the same as the one used in Lab 1 for compiling the C code, except now there are no “rm -f main.s” (clean:).

As in Lab 1 with C programs, you can clean the directory of files created during compilation by typing the following at the command prompt:

make clean

If you cleaned the directory, compile the program (main.S) again by typing make at the command prompt. You should now see main.o, program.dis, and program.elf. main.o is the object file and program.elf is the ELF (executable and linkable format) executable that you will use to load the program onto the MIPS core. program.dis shows the disassembly of the executable (boot and program code). It is a human-readable version of the ELF executable file. Open program.dis using a text editor. The top of the file shows the boot code, starting at 0x9fc00000. This code will be placed at physical address 0x1fc00000, which is the first instruction fetched upon reset of the MIPS core.

Figure 1 shows the user code from program.txt (or program.dis, with file listing information between code lines deleted).  This file shows the assembly code interweaved with the memory address and machine code for the instruction.

80000204 <main>:

80000204:	3c0cbf80 	lui	t4,0xbf80

80000208:	258d0004 	addiu	t5,t4,4

8000020c:	00005021 	move	t2,zero

80000210 <readIO>:

80000210:	8daa0000 	lw	t2,0(t5)

80000214:	ad8a0000 	sw	t2,0(t4)

80000218:	1000fffd 	b	80000210 <readIO>

8000021c:	ad8a0010 	sw	t2,16(t4)

80000220:	00000000 	nop

Figure 1. program.txt: Beginning of user code starting at 0x80000204

For example, the 32-bit machine instructions for lui $12, 0xbf80 is 0x3c0cbf80, located at memory address 0x80000204. That machine instruction is followed immediately by the next machine instruction (0x258d0004 – i.e., addiu $13, $12, 4), which is located 4 bytes (32 bits) later at memory address 0x80000208. The next instruction (lw t2, 0(t5) = lw $10, 0($13) = 0x8daa0000) is at memory address 0x8000020c, and so forth. Notice that the compiler placed the sw instruction that writes to the 7-Segment Displays (sw $10, 16($12)) into the branch delay slot at address 0x8000021c, so the nop placed at 0x80000220 is never executed.

- Step 3. Load the program onto MIPS using UART

Now that the example program is compiled, load it onto the MIPS core using the UART.

Assembly Code with Function Calls

You will now run MIPS assembly code with function calls. Browse to the Lab_2\ASM\ReadSwitchesFunctions directory and open main.S (also shown in Figure 2). This program uses two function calls: one to read the switches (peek), and one to write to the LEDs (poke).

# $12 = address of switches, $13 = address offset of switches

# $10 = switch values

.globl main

main:

lui   $12, 0xbf80       # $12 = address of LEDs

addi  $13, $12, 4       # $13 = address of switches

displaySW:

add   $a0, $13, $0      # put address of switches in $a0

jal   peek              # call peek function

nop                     # branch delay slot

add   $a0, $v0, $0      # move the return value into $a0

add   $a1, $12, $0      # move address of LEDs in $a1

jal   poke              # call poke function

nop                     # branch delay slot

beq   $0, $0, displaySW # repeat

nop                     # branch delay slot

peek:

lw    $v0, 0($a0)       # read I/O register value into $v0

jr    $ra               # return to point of call

nop                     # branch delay slot

poke:

sw    $a0, 0($a1)       # write values to LED

sw    $a0, 16($a1)      # write values to 7-Segs

jr    $ra               # return to point of call

nop                     # branch delay slot

Figure 2. ReadSwitchesFunctions MIPS assembly program that uses function calls

The program, which starts just after the main label, loads the addresses of the LEDs and switches into $12 and $13. Then it puts the address of the switches in the 0th argument register ($a0) and calls the peek function using jal peek.

The peek function, located at the peek label, reads the value of the switches and places that value in the return register ($v0). The function completes by returning to the place it was called, using jr $ra.

The main function then places the value of the switches into $a0 and the address of the LEDs into $a1 and calls the poke function. The poke function writes $a0 to the LEDs.

Compile and load the ReadSwitchesFunctions program using make as described above. Remember that you can also view the program.dis file to see how the MIPS assembly code was compiled.

Fibonacci Numbers

Now use the ReadSwitchesFunctions example MIPS assembly program as a starting point to write your own program. You will write a program that will calculate and display the first 11 Fibonacci numbers; however, this time you will write it using MIPS assembly language.

Make a copy of the Lab_2\ASM\ReadSwitches folder and rename the new folder Fibonacci. Write your program in the main.S file in the Fibonacci folder. Your program should compute the Fibonacci numbers for n = 1…11 and output each Fibonacci number to the LEDs. The LEDs should display the Fibonacci numbers in binary, with a delay between each number so that they are viewable.

Once you have finished writing your program, use the make command to compile it. If there are any errors, fix them and recompile.

After your Fibonacci program compiles without errors, load it onto the MIPS core, run it, and confirm that it works as expected.